uBuilder — microBuilder

Project Summary & Handoff Document

Version 0.1 — March 2026

# 1. Project Overview

uBuilder (microBuilder) is a lightweight browser-based tool for designing, generating and executing C code for microcontroller MAL (Microcontroller Abstraction Layer) and RAL (Register Abstraction Layer). The tool is microcontroller-agnostic: users design their own MAL/RAL for any target hardware.

## 1.1 Key Concepts

| Term | Description |
| --- | --- |
| MAL | Microcontroller Abstraction Layer — memory segments mapped to hardware |
| RAL | Register Abstraction Layer — register definitions and bitfields |
| Segment | A named memory region with start address, size, fill byte and memory type |
| Register | A named RAL register with type (register, bitfield, register_group, peripheral_block) |

# 2. System Requirements

## 2.1 Functional Requirements

- FR1 — Design, generate and execute C code for MAL and RAL

- FR2 — Generate code for real target (bare metal) and PC simulation mode

- FR3 — Hybrid simulation architecture: microcontroller core on remote cluster (MAL/RAL as services); GUI client with stubbed API for remote communication

- FR4 — Export MAL/RAL to YAML file; import MAL/RAL from YAML

- FR5 — Save and load the full design environment

- FR6 — Browser-based GUI, no external dependencies, single HTML page (SPA)

## 2.2 GUI Requirements

| ID | Requirement |
| --- | --- |
| GR1 | Single HTML page (SPA), no external dependencies |
| GR2 | Layout: Toolbar (top) / Left Panel tree view / Main Canvas / Status Bar (bottom) |
| GR3 | Toolbar: New, Save, Load project; Export/Import YAML; Generate C (target + simulation) |
| GR4 | Left panel: tree view with MAL and RAL root nodes; add/delete nodes |
| GR5 | Canvas: drag-and-drop cards, snap-to-grid toggle, no graphical overlap between cards |
| GR6 | Properties panel: inline editing of all node properties |
| GR7 | Status bar: project name, MAL/RAL node count, operation messages |
| GR8 | Style: Neutral Dark Blue — JetBrains IDE aesthetic, JetBrains Mono + Inter fonts |

# 3. MAL Segment Specification

Each MAL entry represents a memory segment with the following properties:

| Field | Type | Description |
| --- | --- | --- |
| Segment ID | string | Unique identifier — duplicates rejected |
| Memory Type | enum | RAM / ROM / FLASH / PERIPH |
| Start Address | hex string | e.g. 0x40020000 |
| Size | dec or hex | e.g. 256 or 0x100 — overlap with other segments rejected |
| End Address | computed | Displayed read-only: start + size - 1 |
| Fill byte | hex string | Default byte value for uninitialised memory, e.g. 0x00 |
| Description | string | Free text description |

## 3.1 Memory Type Colors

| Type | Border color | Meaning |
| --- | --- | --- |
| RAM | Yellow #FFD600 | Read/write volatile memory |
| ROM | Red #EF5350 | Read-only non-volatile memory |
| FLASH | Red #EF5350 | Flash non-volatile memory |
| PERIPH | Green #66BB6A | Peripheral register region |

When a segment card is selected, the border turns bright white (#FFFFFF) regardless of memory type.

# 4. RAL Register Specification

Each RAL entry represents a hardware register with the following properties:

| Field | Type | Description |
| --- | --- | --- |
| Name | string | Unique identifier — duplicates rejected |
| Type | enum | register / bitfield / register_group / peripheral_block |
| Description | string | Free text description |

# 5. Validation Rules

- ID uniqueness: all node names (MAL segments and RAL registers) must be unique across the entire project

- Memory overlap: two MAL segments cannot have overlapping address ranges [addr, addr+size)

- Both validations apply at creation time (modal) and during inline editing (Properties panel)

- Errors are shown inline under the relevant field in the modal; in the Properties panel they appear in the status bar

# 6. Canvas Features

- Cards are draggable by their header bar

- Snap to grid: toggleable button in canvas toolbar; grid size = 32px

- No graphical overlap: during drag, cards are automatically pushed away from other cards (minimum 4px gap)

- Cards display all key segment/register info inline

# 7. File Formats

## 7.1 Project file (.ubp)

JSON format containing: project name, nodes array (full node objects), nextId counter.

## 7.2 YAML export format

Separate mal: and ral: sections. MAL entries include: name, memType, segAddr, segSize, segDefault, desc. RAL entries include: name, type, desc.

# 8. C Code Generation

| Mode | Output | Notes |
| --- | --- | --- |
| Target | .h header file | RAL structs + MAL init/deinit/configure prototypes |
| Simulation | .h header file | Same as target + #include "ubuilder_sim.h" stub header |

# 9. Planned Architecture (Future)

The full system will use a hybrid client/server architecture for simulation mode:

| Component | Technology | Role |
| --- | --- | --- |
| GUI Client | Vanilla HTML/CSS/JS (SPA) | Design tool, runs in browser |
| Backend Server | Python + FastAPI | MAL/RAL as services, microcontroller core emulation |
| Code Generator | Python | Generates C code from project state |
| Communication | REST + WebSocket | Real-time GUI ↔ server sync |

# 10. Current Implementation Status

| Feature | Status |
| --- | --- |
| SPA GUI layout (toolbar, tree, canvas, properties, status bar) | ✅ Done |
| MAL segment add/edit/delete with full validation | ✅ Done |
| RAL register add/edit/delete with full validation | ✅ Done |
| ID uniqueness check (MAL + RAL) | ✅ Done |
| Memory overlap check (address range) | ✅ Done |
| Memory type colors (RAM/ROM/FLASH/PERIPH) | ✅ Done |
| End address + hex size display on cards | ✅ Done |
| Drag and drop cards on canvas | ✅ Done |
| Snap to grid (32px) | ✅ Done |
| No graphical card overlap during drag | ✅ Done |
| Save/Load project (.ubp JSON) | ✅ Done |
| Export/Import YAML | ✅ Done |
| Basic C code generation (target + simulation) | ✅ Done |
| Backend server (Python/FastAPI) | ⬜ Not started |
| MAL/RAL as remote services | ⬜ Not started |
| Simulation mode stubs | ⬜ Not started |
| Visual connections between nodes | ⬜ Not started |

# 11. How to Resume This Session

To continue development with a new Claude session:

- Open a new Claude chat

- Upload both files: this document (uBuilder_Summary.docx) and the current index.html

- Write: "Stiamo sviluppando uBuilder. Allego il documento di riepilogo e il codice attuale. Voglio continuare da dove eravamo."

Claude will read the summary document and the current index.html and will be able to continue seamlessly.

uBuilder — microBuilder  |  v0.1  |  March 2026